# TPG → Scaler Configuration Sequence

Nios V · Video & Vision Processing Suite (HAL drivers) · reference for software/app/hello.c

This document lists the order of driver API calls used to bring up the video pipeline, and the hardware register each call reads or writes. Read top-to-bottom: the scaler is configured first, then the TPG is started last.

[ TPG @ 0x30400 ] → [ Scaler @ 0x30000 ] → [ Exported to video out ]

| Access tag | Meaning |
| --- | --- |
| WRITE | A configuration value is written to the register. |
| READ | The register is read back from hardware. |
| CACHED | Helper returns a value cached in the instance struct — no bus access. |

## 1. TPG — Test Pattern Generator

Function: setup_tpg() · base address 0x30400 · the video source.

| Step | API call | Access | Register(s) | Notes |
| --- | --- | --- | --- | --- |
| 1 | intel_vvp_tpg_init(tpg, TPG_0_BASE) | READ + WRITE | core VENDOR/PRODUCT_ID, core REGMAP_VERSION, LITE_MODE_REG, DEBUG_ENABLED_REG, NUM_PATTERNS_REG, BPS_REG, PIP_REG; then CONTROL_REG = 0 | Validates the IP, caches compile-time params, and stops the core. |
| 2 | get_num_patterns / get_bits_per_sample / get_pixels_in_parallel / get_lite_mode | CACHED | none | Diagnostic printf only; returns values cached by init(). |
| 3 | intel_vvp_tpg_stop(tpg) | WRITE | CONTROL_REG = 0 | Clears the GO bit so the source is idle while reconfiguring. |
| 4 | intel_vvp_core_set_img_info_width / _height(tpg, …) | WRITE | core IMG_INFO_WIDTH, core IMG_INFO_HEIGHT | Output frame size. Lite mode only (full mode emits size in-band). |
| 5 | intel_vvp_tpg_set_pattern(tpg, 0) | WRITE | PATTERN_SELECT_REG | Selects pattern slot 0 (slot contents fixed at IP-gen time). |
| 6 | intel_vvp_tpg_set_bars_type(tpg, kIntelVvpTpgColorBars) | WRITE | BARS_SELECT_REG | Color / grey / B&W / mixed bar style. |
| 7 | intel_vvp_tpg_commit_writes(tpg) | WRITE | COMMIT_REG | Latches staged writes. Full mode only (immediate in lite mode). |
| 8 | intel_vvp_tpg_start(tpg) | WRITE | CONTROL_REG = GO_MSK | Sets the GO bit — TPG begins generating frames. |

## 2. Scaler

Function: setup_scaler() · base address 0x30000 · has no start/stop; runs once configured.

| Step | API call | Access | Register(s) | Notes |
| --- | --- | --- | --- | --- |
| 1 | intel_vvp_scaler_init(scaler, SCALER_0_BASE) | READ | core VENDOR/PRODUCT_ID, core REGMAP_VERSION, LITE_MODE_REG, DEBUG_ENABLED_REG, PIXELS_IN_PARALLEL_REG, MAX_INPUT_WIDTH_REG, MAX_OUTPUT_WIDTH_REG, ALGORITHM_REG; then (per algorithm) V/H_SCALING_ENABLED, V/H_NUM_TAPS, V/H_NUM_PHASES, V/H_NUM_BANKS, V/H_COEFFS_* | Validates the IP and caches every compile-time parameter. No writes. |
| 2 | is_horizontal/vertical_scaling_enabled / get_max_* / get_lite_mode | CACHED | none | Diagnostic printf and the guards on steps below. |
| 3 | intel_vvp_core_set_img_info_width / _height(scaler, …) | WRITE | core IMG_INFO_WIDTH, core IMG_INFO_HEIGHT | Scaler INPUT size (= TPG output). Lite mode only (full mode in-band). |
| 4 | intel_vvp_scaler_set_output_width(scaler, out_w) | WRITE | OUTPUT_WIDTH_REG | Only if H-scaling enabled. Rejected if 0 or > MAX_OUTPUT_WIDTH. |
| 5 | intel_vvp_scaler_set_output_height(scaler, out_h) | WRITE | OUTPUT_HEIGHT_REG | Only if V-scaling enabled. Rejected if 0. |
| 6 | intel_vvp_scaler_commit_writes(scaler) | WRITE | COMMIT_REG | Latches output-size writes. Full mode only. No start call follows. |

Why scaler-first, TPG-last: the scaler has no enable bit, so it must be fully configured before any pixels reach it. main() calls setup_scaler() then setup_tpg(), and only the final tpg_start() sets pixels flowing into an already-ready scaler.

### Register reference — written during configuration

| IP | Register | Access | Configured by | Purpose |
| --- | --- | --- | --- | --- |
| TPG | CONTROL_REG | WRITE | stop() / start() | GO bit — 0 = idle, GO_MSK = generating |
| TPG | core IMG_INFO_WIDTH / _HEIGHT | WRITE | core_set_img_info_* | Output frame size (lite mode) |
| TPG | PATTERN_SELECT_REG | WRITE | set_pattern | Active pattern slot index |
| TPG | BARS_SELECT_REG | WRITE | set_bars_type | Color / grey / B&W / mixed bars |
| TPG | COMMIT_REG | WRITE | commit_writes | Latch staged writes (full mode) |
| TPG | STATUS_REG | READ | is_running | RUNNING / PENDING_COMMIT flags |
| Scaler | core IMG_INFO_WIDTH / _HEIGHT | WRITE | core_set_img_info_* | Input frame size (lite mode) |
| Scaler | OUTPUT_WIDTH_REG | WRITE | set_output_width | Target output width (H-scaling) |
| Scaler | OUTPUT_HEIGHT_REG | WRITE | set_output_height | Target output height (V-scaling) |
| Scaler | COMMIT_REG | WRITE | commit_writes | Latch output-size writes (full mode) |
| Scaler | STATUS_REG | READ | is_running | RUNNING / PENDING_COMMIT flags |

Source: software/app/hello.c · drivers intel_vvp_tpg / intel_vvp_scaler / intel_vvp_core. All config-time register reads happen once inside each init(); the get_* query helpers return cached struct fields.